EIA activity 1

13.What are the direct effects of EMF exposure on the human reproductive outcome (100 kHz – 300 GHz)? Are the studies conclusive? (G6 – G41 – G27 – G14)

Studies on reproductive outcomes for women show different results

- There are two extensive studies on women treated with microwave diathermy during labor to relieve the pain of uterine contractions and no evidence was found for harmful effects on the fetus.

- seven studies on pregnancy outcomes among workers occupationally exposed to microwave radiation and on birth defects among their offspring produced both positive and negative results.

- In some of the larger epidemiological studies of female plastic welders and physiotherapists working with shortwave diathermy devices, there were no statistically significant effects on rates of abortion or fetal malformation.

- other studies on similar populations of female workers found an increased risk of miscarriage and birth defects.

- A study shows that there is no association between microwave exposure and the risk of Down’s syndrome in the offspring of male radar workers.

From the studies shown above we can conclude that these studies are not conclusive about the risks of microwave exposure. Despite the generally negative results of these studies, it will be difficult to draw firm conclusions on reproductive risk without further epidemiological data on highly exposed individuals and more precise exposure assessment.

Reference

ICNIRP_Guide RF, page 12

22. What are the SAR limits as basic restrictions according to Canada Safety Code 6? (G15 – G6 – G12)

The specific absorption rate (SAR) is a measure of the rate at which electromagnetic energy is absorbed in the body its measuring unit is (W/kg). At frequencies between 100 kHz and 6 GHz, SAR limits have the priority over field strength and power density limits and shall not be exceeded.

The SAR should be determined for situations where exposures occur at a distance of 0.2 m or less from the source.

When SAR determination is practical and can be achieved then the values in the following table shouldn’t be exceeded.

| Condition | SAR limit (W/kg) Controlled environment | SAR limit (W/kg) uncontrolled environment |
| --- | --- | --- |
| The SAR averaged over the whole body mass. | 0.4 | 0.08 |
| The spatial peak SAR for the head, neck and trunk, averaged over any one gram (g) of tissue*. | 8 | 1.6 |
| The spatial peak SAR in the limbs as averaged over any 10 g of tissue*. | 20 | 4 |

*1g of tissue represents an approximate volume of 1cm3

Although not a requirement of the code, it is suggested that whenever possible, the organ-averaged SAR for the eye should not exceed 0.4 W/kg in the controlled environment and 0.2 W/kg in the uncontrolled environment.